Tertiary Infotech Academy Pte Ltd

UEN: 201200696W

LEARNER GUIDE

For

CompTIA Certified Network+ Training (N10-009)

TGS Ref No: TGS-2025054472

Conducted by

Tertiary Infotech Academy Pte Ltd

UEN: 201200696W

Version 1.0

DOCUMENT VERSION CONTROL RECORD

| Version Number | Effective Date of Release | Summary of Included Changes | Author |
| --- | --- | --- | --- |
| 1.0 | 2 July 2026 | Initial release — Network+ (N10-009) Learner Guide covering all 36 labs. | Dr. Alfred Ang |

TABLE OF CONTENTS

Right-click and choose “Update Field”, or press F9, to build the table of contents.

# Introduction

This Learner Guide accompanies the WSQ course CompTIA Certified Network+ Training (N10-009) (TGS-2025054472), conducted by Tertiary Infotech Academy Pte Ltd. It provides step-by-step instructions for all 36 hands-on labs, organised by the five official CompTIA Network+ N10-009 exam domains. Every lab maps to a published exam objective and is completed on the free Killercoda Ubuntu Playground in your browser.

Use this guide alongside the course slides and the lab files in the labs/ folder of the course repository. Reset the Killercoda playground between labs that change firewall or routing rules so each lab starts from a clean state.

# Course Learning Outcomes

LO1: Explain networking concepts — the OSI model, IPv4/IPv6 addressing and subnetting, ports and protocols, DNS and DHCP, network topologies, architectures and cloud connectivity.

LO2: Implement networks — static and dynamic routing, 802.1Q VLANs, trunking, link aggregation, NAT/PAT, wireless, and core DHCP and DNS services.

LO3: Operate and maintain networks — SNMP monitoring, flow analysis, centralized logging and SIEM, time synchronization, configuration backup and performance baselining.

LO4: Secure networks — firewalls, ACLs, IDS/IPS, TLS certificates, SSH hardening, VPNs, port scanning and access control.

LO5: Troubleshoot networks — apply the CompTIA seven-step methodology using layer-by-layer diagnostics, connectivity tools and packet capture.

# Before You Start — Environment Setup

What you need

A computer with a modern web browser and internet access.

The free Killercoda Ubuntu Playground — https://killercoda.com/playgrounds/scenario/ubuntu (no local install, no login required).

For Lab 11 (Wireless): a device with a real Wi-Fi radio and a Wi-Fi analyzer app.

For Lab 32 (Packet Capture): the browser-based TIS PCAP Analyzer — https://alfredang.github.io/pcapanalyzer/.

Launch the Killercoda playground

Open the playground URL in a browser tab. You get a disposable Ubuntu virtual machine with a root shell — nothing to install. Most labs install their own tools with apt at the first step.

apt update            # refresh the package index

ip -brief addr        # confirm your interface (usually eth0) and IP

Conventions used in every lab

Run the shell commands on the Killercoda terminal unless a step says to use a web tool.

Commands are shown exactly as you type them; output samples follow in the lab files.

Reset the playground (or run the lab's clean-up step) between labs that change firewall, routing or interface state.

A subnet/IP calculator is available at https://alfredang.github.io/ipcalculator/ for the addressing labs.

# Domain 1.0 — Networking Concepts  (23%)

OSI Model · IPv4/IPv6 Addressing · Ports & Protocols · DNS & DHCP · Topologies & Cloud

Key concepts

The OSI model's seven layers (Physical → Application) describe how data is encapsulated and de-encapsulated as it crosses a network.

IPv4 uses 32-bit dotted-decimal addresses with CIDR subnet masks; IPv6 uses 128-bit hexadecimal addresses with SLAAC and link-local addressing.

Ports and protocols (TCP vs UDP, well-known ports 20–443) identify services; DNS resolves names and DHCP leases addresses via DORA.

Modern networks span LAN/WAN topologies, three-tier and spine-leaf architectures, and cloud VPCs with public and private subnets.

## Lab 1 — OSI Model Packet Inspection

Exam objective: inspect packets and map headers to OSI layers.

Goal: The learner captures live traffic with tcpdump and maps each protocol header to its OSI layer.

What you'll build

A packet capture annotated with the OSI layer of each header (L2 MAC, L3 IP, L4 TCP, L7 HTTP)   (Tools & protocols: tcpdump, curl, ip, Berkeley Packet Filter.)

Step-by-step

Install the capture tools

apt update && apt install -y tcpdump curl

Identify the active interface

ip -brief addr

Start a background capture on TCP 80

tcpdump -i eth0 -nn -v -c 5 'tcp port 80' &

Generate HTTP traffic

curl -s http://example.com > /dev/null

Read the packet and map it to the OSI model

tcpdump -i eth0 -nn -e -A -c 10 'tcp port 80' &

Stop background captures

kill %1 2>/dev/null

Test it

Verify you can point at a captured packet and name the OSI layer of each header.

Note: Full commands and expected output are in labs/lab-01-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 2 — IPv4 Subnetting and CIDR

Exam objective: subnet a network into CIDR blocks and verify boundaries.

Goal: The learner splits a /24 network into four /26 subnets, verifies the math in the TIS IP Calculator, and assigns a CIDR address on Linux.

What you'll build

Four /26 subnets with verified network, host range and broadcast, plus an assigned interface IP   (Tools & protocols: TIS IP Calculator, ip, CIDR.)

Step-by-step

Open the IP Calculator at https://alfredang.github.io/ipcalculator/

Understand the parent network 192.168.10.0/24 in the calculator

Split into four /26 subnets using the calculator

Assign an IP from subnet 2 to your interface

ip addr add 192.168.10.65/26 dev eth0 label eth0:1

Verify the address is applied

ip -4 addr show eth0

Clean up the assigned address

ip addr del 192.168.10.65/26 dev eth0

Test it

Verify a /24 divides into four /26 networks each with 62 usable hosts and a unique broadcast.

Note: Full commands and expected output are in labs/lab-02-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 3 — IPv6 Addressing and Link-Local

Exam objective: plan IPv6 prefixes and distinguish link-local from global addresses.

Goal: The learner plans an IPv6 prefix in the TIS IP Calculator, adds a global address, observes the auto-generated link-local address, and pings across both.

What you'll build

An interface carrying both a link-local fe80::/10 address and a global 2001:db8::10/64 address   (Tools & protocols: TIS IP Calculator, ip, ping6, SLAAC.)

Step-by-step

Plan the IPv6 prefix 2001:db8::/64 in the calculator at https://alfredang.github.io/ipcalculator/

Inspect the existing IPv6 configuration

ip -6 addr show eth0

Add a documentation-range global address

ip -6 addr add 2001:db8::10/64 dev eth0

Verify the new address

ip -6 addr show eth0

Ping the all-nodes link-local multicast

ping6 -c2 ff02::1%eth0

Ping the global address from itself

ping6 -c2 2001:db8::10

Clean up the global address

ip -6 addr del 2001:db8::10/64 dev eth0

Test it

Verify you can distinguish a link-local fe80::/10 address from a global address and ping each.

Note: Full commands and expected output are in labs/lab-03-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 4 — Common Ports and Protocols

Exam objective: map common protocol names to their well-known ports.

Goal: The learner enumerates listening services and scans a live target to connect protocol names to their well-known port numbers.

What you'll build

A verified mapping of protocol names to well-known ports from local listeners and a remote scan   (Tools & protocols: nmap, ss, net-tools, getent.)

Step-by-step

Install nmap and net-tools

apt update && apt install -y nmap net-tools

List ports listening on your own host

ss -tulnp

Scan a public test target

nmap -sT -p 1-1024 scanme.nmap.org

Map the results to the exam port list

Resolve a service name to a port

getent services https

Test it

Verify you can name the well-known port for each common protocol tested on N10-009.

Note: Full commands and expected output are in labs/lab-04-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 5 — DNS Record Types

Exam objective: query each DNS record type on the exam blueprint.

Goal: The learner uses dig to query every DNS record type in the N10-009 objectives: A, AAAA, MX, TXT, NS, CNAME, PTR, SOA, and SRV.

What you'll build

A set of dig query results covering A, AAAA, MX, TXT, NS, CNAME, PTR, SOA and SRV records   (Tools & protocols: dig, dnsutils, DNS.)

Step-by-step

Install dnsutils

apt update && apt install -y dnsutils

Query an A record

dig +short A google.com

Query an AAAA record

dig +short AAAA google.com

Query an MX record

dig +short MX google.com

Query a TXT record

dig +short TXT google.com

Query NS records

dig +short NS google.com

Query a CNAME record

dig +short CNAME www.github.com

Reverse lookup a PTR record

dig +short -x 8.8.8.8

Query the SOA record

dig +short SOA google.com

Query an SRV record

dig +short SRV _xmpp-server._tcp.gmail.com

Test it

Verify you can query each required record type and explain how a CNAME differs from an A record.

Note: Full commands and expected output are in labs/lab-05-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 6 — DHCP Lease Capture (DORA)

Exam objective: capture the four-step DHCP DORA exchange.

Goal: The learner releases an existing DHCP lease and captures the four DORA messages (Discover, Offer, Request, Acknowledge) live with tcpdump.

What you'll build

A live capture showing DHCPDISCOVER, DHCPOFFER, DHCPREQUEST and DHCPACK on UDP 67/68   (Tools & protocols: isc-dhcp-client, dhclient, tcpdump, DHCP.)

Step-by-step

Install the tools

apt update && apt install -y isc-dhcp-client tcpdump

Start a capture for DHCP ports

tcpdump -i eth0 -nn -v 'port 67 or port 68' &

Release the current lease

dhclient -v -r eth0

Request a new lease

dhclient -v eth0

Stop the capture

kill %1 2>/dev/null

Test it

Verify you can identify all four DHCP DORA messages and the UDP ports (67/68) they use.

Note: Full commands and expected output are in labs/lab-06-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 7 — Cloud VPC with Network Namespaces

Exam objective: simulate a cloud VPC using Linux network namespaces.

Goal: The learner builds a VPC with two subnets and a bridge using Linux network namespaces and veth pairs, then tests connectivity between the two virtual machines.

What you'll build

A simulated VPC with two namespaces on a shared /24 subnet connected through a Linux bridge   (Tools & protocols: iproute2, ip netns, veth, bridge.)

Step-by-step

Create two namespaces representing two VMs

ip netns add web

Create a bridge as the VPC switch

ip link add br0 type bridge

Connect each namespace to the bridge with a veth pair

ip link add veth-web type veth peer name br-web

Assign IPs in the same /24 subnet

ip netns exec web ip addr add 10.0.1.10/24 dev veth-web

Test connectivity between the two VMs

ip netns exec web ping -c2 10.0.1.20

Clean up namespaces and bridge

ip netns del web

Test it

Verify two hosts on the same subnet can talk through a bridge with no router.

Note: Full commands and expected output are in labs/lab-07-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

# Domain 2.0 — Network Implementation  (20%)

Routing · VLANs & Trunking · Link Aggregation · NAT/PAT · Wireless · DHCP & DNS Services

Key concepts

Routing moves packets between subnets using static routes and dynamic protocols (RIP, OSPF, EIGRP, BGP) ranked by administrative distance.

Switching features — 802.1Q VLANs, trunking, link aggregation (LACP), STP and port security — segment and protect the LAN.

NAT and PAT translate private addresses to public; a DHCP server and authoritative DNS (BIND9) provide core network services.

Wireless networks use the 2.4/5/6 GHz bands, non-overlapping channels, SSID/BSS/ESS and 802.1X for secure access.

## Lab 8 — Static Routing Between Subnets

Exam objective: configure static routes between subnets.

Goal: The learner builds a three-node subnet A - router - subnet B topology and adds the static routes that let the two subnets communicate.

What you'll build

A routed topology with a forwarding router and static routes on each end host.   (Tools & protocols: iproute2, network namespaces, veth pairs, sysctl ip_forward.)

Step-by-step

Create three network namespaces

ip netns add a

Wire A-R1 and B-R1 with veth pairs

ip link add a-r type veth peer name r-a

Assign IPs in two different subnets

ip netns exec a  ip addr add 10.1.1.2/24 dev a-r

Enable IP forwarding on the router

ip netns exec r1 sysctl -w net.ipv4.ip_forward=1

Add static routes on the end hosts

ip netns exec a ip route add 10.2.2.0/24 via 10.1.1.1

Verify connectivity across subnets

ip netns exec a ping -c2 10.2.2.2

Clean up the namespaces

for n in a r1 b; do ip netns del $n; done

Test it

A ping from subnet A to 10.2.2.2 in subnet B succeeds, proving the static routes forward traffic through the router.

Note: Full commands and expected output are in labs/lab-08-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 9 — 802.1Q VLAN Tagging

Exam objective: create 802.1Q VLAN sub-interfaces.

Goal: The learner loads the 802.1Q kernel module and creates two tagged VLAN sub-interfaces on Linux to separate broadcast domains.

What you'll build

Two VLAN sub-interfaces (eth0.10 and eth0.20) each on its own subnet.   (Tools & protocols: iproute2, 8021q kernel module, tcpdump.)

Step-by-step

Load the 802.1Q kernel module

modprobe 8021q

Create two VLAN sub-interfaces on eth0

ip link add link eth0 name eth0.10 type vlan id 10

Assign each VLAN its own subnet

ip addr add 192.168.10.1/24 dev eth0.10

Verify the VLAN IDs on the sub-interfaces

ip -d link show eth0.10

Capture a tagged frame with tcpdump

tcpdump -i eth0 -nn -e vlan &

Clean up the VLAN sub-interfaces

ip link del eth0.10

Test it

tcpdump prints the 802.1Q VLAN tag on frames sent through eth0.10, confirming the frames are tagged with VLAN 10.

Note: Full commands and expected output are in labs/lab-09-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 10 — Link Aggregation (LACP Bond)

Exam objective: bundle NICs into an LACP bond.

Goal: The learner installs ifenslave, loads the bonding driver in 802.3ad mode and creates an LACP bond interface with an enslaved NIC.

What you'll build

A bond0 LACP (802.3ad) interface with an enslaved eth0 slave.   (Tools & protocols: ifenslave, bonding kernel module, LACP (IEEE 802.3ad).)

Step-by-step

Install ifenslave and load the bonding driver

apt update && apt install -y ifenslave

Create the bond interface in 802.3ad mode

ip link add bond0 type bond mode 802.3ad

Add slave interfaces to the bond

ip link set eth0 master bond0

Inspect the bond status

cat /proc/net/bonding/bond0

Clean up the bond and slave

ip link del bond0

Test it

Reading /proc/net/bonding/bond0 shows the LACP partner MAC, aggregator ID and per-slave link state, confirming the bond is active.

Note: Full commands and expected output are in labs/lab-10-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 11 — Wireless Concepts and Survey

Exam objective: survey wireless bands, channels and security.

Goal: The learner surveys wireless bands and channels with iw and compares 2.4/5/6 GHz behaviour and WPA security modes.

What you'll build

A basic Wi-Fi site survey plus a channel-to-band and security-mode reference.   (Tools & protocols: iw, wavemon, WiFi Analyzer, Wireless Diagnostics.)

Step-by-step

List supported PHYs on a wireless device

iw list | less

Scan nearby access points

iw dev wlan0 scan | grep -E "SSID|freq|signal|RSN|WPA"

Map non-overlapping channels to bands

Compare WEP, WPA2 and WPA3 security modes

Quiz yourself on the wireless concepts

Test it

The learner records SSID, channel, signal and security mode for nearby APs and correctly maps channels to the 2.4/5/6 GHz bands.

Note: Full commands and expected output are in labs/lab-11-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 12 — NAT and PAT with iptables

Exam objective: translate private IPs with NAT and PAT.

Goal: The learner configures an iptables MASQUERADE rule so hosts on a private subnet share the host public IP via port address translation.

What you'll build

A MASQUERADE (PAT) rule on the host translating 10.1.1.0/24 out eth0.   (Tools & protocols: iptables, network namespaces, MASQUERADE, conntrack.)

Step-by-step

Re-create the private namespace and host link

ip netns add a

Enable forwarding on the host

sysctl -w net.ipv4.ip_forward=1

Add the MASQUERADE rule for PAT

iptables -t nat -A POSTROUTING -s 10.1.1.0/24 -o eth0 -j MASQUERADE

Test outbound access from the namespace

ip netns exec a ping -c2 8.8.8.8

Inspect the conntrack translation table

conntrack -L | head

Clean up rules, namespace and link

iptables -t nat -F POSTROUTING

Test it

curl ifconfig.me from inside the namespace returns the host public IP rather than 10.1.1.2, proving the source address was translated.

Note: Full commands and expected output are in labs/lab-12-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 13 — DHCP Server (isc-dhcp-server)

Exam objective: configure a DHCP scope and lease server.

Goal: The learner installs isc-dhcp-server and configures a DHCP scope on a dummy interface to hand out leases on a private subnet.

What you'll build

A running ISC DHCP server with a 10.50.0.0/24 scope on a dummy interface.   (Tools & protocols: isc-dhcp-server, dummy interface, systemctl.)

Step-by-step

Install the ISC DHCP server

apt update && apt install -y isc-dhcp-server

Create a dummy interface for the lab

ip link add dum0 type dummy

Configure the DHCP scope

cat >/etc/dhcp/dhcpd.conf <<EOF

Set the interface the server listens on

sed -i 's/^INTERFACESv4=.*/INTERFACESv4="dum0"/' /etc/default/isc-dhcp-server

Start the DHCP service

systemctl restart isc-dhcp-server

Inspect the leases file

cat /var/lib/dhcp/dhcpd.leases

Clean up the service and interface

systemctl stop isc-dhcp-server

Test it

systemctl status isc-dhcp-server shows the daemon running and bound to dum0 with the configured scope.

Note: Full commands and expected output are in labs/lab-13-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 14 — Authoritative DNS with BIND9

Exam objective: serve an authoritative DNS zone.

Goal: The learner runs a BIND9 server hosting the authoritative zone lab.local and queries its records with dig.

What you'll build

A BIND9 master server serving the lab.local zone with A, NS and MX records.   (Tools & protocols: bind9, dnsutils (dig), named-checkzone.)

Step-by-step

Install BIND9 and dnsutils

apt update && apt install -y bind9 dnsutils

Create the lab.local zone file

cat >/etc/bind/db.lab.local <<'EOF'

Register the zone with BIND9

cat >>/etc/bind/named.conf.local <<'EOF'

Validate the zone and reload BIND9

named-checkzone lab.local /etc/bind/db.lab.local

Query your own DNS server with dig

dig @127.0.0.1 www.lab.local

Clean up by stopping BIND9

systemctl stop bind9

Test it

dig @127.0.0.1 www.lab.local returns the configured A record, confirming BIND9 answers authoritatively for lab.local.

Note: Full commands and expected output are in labs/lab-14-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

# Domain 3.0 — Network Operations  (19%)

SNMP · NetFlow · Syslog & SIEM · NTP · Config Backup · Performance Baselines

Key concepts

SNMP (traps, MIBs, v3), NetFlow flow data and packet capture give visibility into device health and traffic.

Centralized logging (syslog / rsyslog) and SIEM correlate events; NTP keeps timestamps consistent across devices.

Configuration management relies on backups and golden / baseline configs stored in version control (Git).

Performance baselines (iperf3), documentation (physical / logical diagrams, IPAM) and SLAs keep the network reliable.

## Lab 15 — SNMP Monitoring

Exam objective: monitor devices with SNMP.

Goal: The learner installs an SNMP agent, configures a read-only community string, and walks the MIB to read device metrics.

What you'll build

A working snmpd agent with a read-only community, queried via snmpwalk and snmpget   (Tools & protocols: SNMP, snmpd, snmpwalk, snmpget, IF-MIB.)

Step-by-step

Install agent and client

apt update && apt install -y snmpd snmp snmp-mibs-downloader

Configure a read-only community

echo 'rocommunity public 127.0.0.1' >> /etc/snmp/snmpd.conf

Walk system info

snmpwalk -v2c -c public 127.0.0.1 system

Get a single OID

snmpget -v2c -c public 127.0.0.1 sysUpTime.0

Walk interface counters

snmpwalk -v2c -c public 127.0.0.1 IF-MIB::ifDescr

Test it

Running snmpwalk against the agent returns system OIDs such as sysDescr, sysName, and sysUpTime.

Note: Full commands and expected output are in labs/lab-15-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 16 — NetFlow Analysis with nfdump

Exam objective: export and analyse network flows.

Goal: The learner runs a NetFlow exporter and collector on one VM and analyses the captured flows with nfdump.

What you'll build

A NetFlow exporter (fprobe) feeding a collector (nfcapd) whose flows are reported with nfdump   (Tools & protocols: NetFlow, fprobe, nfcapd, nfdump.)

Step-by-step

Install the tools

apt update && apt install -y nfdump fprobe

Start the collector

nfcapd -p 9995 -l /tmp/flows -D

Start the exporter

fprobe -i eth0 127.0.0.1:9995

Generate traffic

curl -s https://example.com > /dev/null

Analyse the flows

nfdump -R /tmp/flows -s srcip

Clean up

pkill fprobe

Test it

nfdump -s builds top-N reports of source IPs and destination ports from the captured flows.

Note: Full commands and expected output are in labs/lab-16-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 17 — Centralised Logging (rsyslog)

Exam objective: centralise logs with syslog forwarding.

Goal: The learner configures rsyslog to both send and receive on loopback, proving the log-forwarding pipeline.

What you'll build

An rsyslog TCP receiver on port 514 with a forwarding rule that delivers test events   (Tools & protocols: rsyslog, imtcp, logger, syslog TCP/514.)

Step-by-step

Enable the TCP receiver

sed -i 's/#module(load="imtcp")/module(load="imtcp")/' /etc/rsyslog.conf

Add a forwarding rule

echo '*.* @@127.0.0.1:514' > /etc/rsyslog.d/50-forward.conf

Restart rsyslog

systemctl restart rsyslog

Generate a test event

logger -p local0.info "Network+ Lab 17 test event"

Verify the event landed

grep "Lab 17 test" /var/log/syslog

Inspect severity and facility

Test it

grep finds the injected test event in /var/log/syslog after it traverses the forwarding pipeline.

Note: Full commands and expected output are in labs/lab-17-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 18 — Time Sync with Chrony

Exam objective: synchronise clocks with NTP.

Goal: The learner installs chrony, inspects its time sources and tracking state, and forces an immediate sync.

What you'll build

A chrony NTP client synchronising to upstream servers with an added extra source   (Tools & protocols: NTP, chrony, chronyc, UDP/123.)

Step-by-step

Install chrony

apt update && apt install -y chrony

List your time sources

chronyc sources -v

Show synchronisation state

chronyc tracking

Add an extra NTP server

echo "server time.cloudflare.com iburst" >> /etc/chrony/chrony.conf

Force an immediate sync

chronyc -a makestep

Test it

chronyc tracking reports the current stratum and clock offset, confirming synchronisation.

Note: Full commands and expected output are in labs/lab-18-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 19 — Configuration Backup with Git

Exam objective: version-control network configuration.

Goal: The learner places /etc under Git version control to gain change history, audit trail, and rollback.

What you'll build

A Git repository in /etc tracking network configs with commit history and a revert   (Tools & protocols: git.)

Step-by-step

Install git

apt update && apt install -y git

Initialise a repo in /etc

git init

Add critical network configs

git add hosts hostname resolv.conf network/ bind/ 2>/dev/null

Make a change and commit it

echo "127.0.1.2 lab-vm" >> /etc/hosts

Inspect the audit trail

git -C /etc log --oneline

Roll back

git -C /etc revert HEAD --no-edit

Test it

git log shows the commit history and git revert restores /etc/hosts to its prior state.

Note: Full commands and expected output are in labs/lab-19-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 20 — Performance Baseline with iperf3

Exam objective: baseline throughput, jitter, and loss.

Goal: The learner runs an iperf3 server and client to measure TCP throughput, parallel streams, UDP jitter/loss, and reverse-mode traffic.

What you'll build

An iperf3 server-and-client pair producing TCP, parallel, UDP, and reverse-mode measurements   (Tools & protocols: iperf3, TCP, UDP.)

Step-by-step

Install iperf3

apt update && apt install -y iperf3

Run a server in the background

iperf3 -s -D

TCP throughput test (10 seconds)

iperf3 -c 127.0.0.1 -t 10

Parallel streams (saturate WAN circuits)

iperf3 -c 127.0.0.1 -P 4 -t 10

UDP test (jitter + loss)

iperf3 -c 127.0.0.1 -u -b 100M -t 10

Reverse mode (server sends to client)

iperf3 -c 127.0.0.1 -R -t 10

Stop the server

pkill iperf3

Test it

The iperf3 client reports Bitrate for TCP tests and Jitter with Lost/Total Datagrams for the UDP test.

Note: Full commands and expected output are in labs/lab-20-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

# Domain 4.0 — Network Security  (14%)

CIA Triad · Firewalls & ACLs · IDS/IPS · TLS · SSH · VPNs · NAC

Key concepts

The CIA triad (confidentiality, integrity, availability) and defence-in-depth guide network security design.

Firewalls, ACLs, screened subnets and IDS/IPS (Suricata) filter and inspect traffic at the perimeter and on the host.

Encryption secures data in transit — TLS certificates, SSH key authentication and IPsec / WireGuard VPNs (site-to-site, client-to-site).

Hardening, port scanning (nmap), network access control and key management reduce the attack surface.

## Lab 21 — Host Firewall with UFW

Exam objective: configure a host firewall.

Goal: The learner configures a default-deny host firewall with UFW, permitting only required inbound services.

What you'll build

A UFW ruleset that denies all inbound traffic by default and selectively allows SSH and a source-restricted MySQL port.   (Tools & protocols: UFW, iptables, SSH, TCP.)

Step-by-step

Install UFW

apt update && apt install -y ufw

Set sane default policies

ufw default deny incoming

Allow SSH on port 22

ufw allow 22/tcp

Enable the firewall and view status

ufw --force enable

Add a source-restricted MySQL rule

ufw allow from 10.0.0.0/8 to any port 3306 proto tcp

Delete a rule by number

ufw delete 1

Disable the firewall when done

ufw disable

Test it

Running ufw status verbose confirms the default-deny stance with only the intended allow rules.

Note: Full commands and expected output are in labs/lab-21-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 22 — TLS Certificate with OpenSSL + Nginx

Exam objective: serve HTTPS with a TLS certificate.

Goal: The learner generates a self-signed X.509 certificate, serves it from nginx, and verifies the TLS handshake.

What you'll build

A self-signed RSA-2048 certificate and an nginx HTTPS server confirmed via a client-side TLS handshake.   (Tools & protocols: OpenSSL, Nginx, curl, TLS, X.509, HTTPS.)

Step-by-step

Install openssl, nginx and curl

apt update && apt install -y openssl nginx curl

Generate a self-signed certificate

openssl req -x509 -newkey rsa:2048 -nodes \

-keyout /etc/ssl/private/lab.key \

-out    /etc/ssl/certs/lab.crt \

-days 365 \

-subj "/CN=lab.local"

Configure nginx for HTTPS

cat >/etc/nginx/sites-available/default <<'EOF'

server {

listen 443 ssl default_server;

server_name _;

ssl_certificate     /etc/ssl/certs/lab.crt;

ssl_certificate_key /etc/ssl/private/lab.key;

root /var/www/html;

index index.html;

}

EOF

systemctl restart nginx

Test HTTPS from the client side

curl -kv https://localhost 2>&1 | head -30

Inspect the certificate

openssl x509 -in /etc/ssl/certs/lab.crt -noout -text | head -20

Test the live handshake with s_client

echo | openssl s_client -connect localhost:443 -servername lab.local 2>/dev/null | head -20

Test it

curl -kv https://localhost shows a successful TLS 1.3 handshake presenting the lab.local certificate.

Note: Full commands and expected output are in labs/lab-22-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 23 — SSH Hardening with Key Auth

Exam objective: harden ssh with key authentication.

Goal: The learner creates an Ed25519 key pair, installs the public key, and disables SSH password authentication.

What you'll build

A key-only SSH configuration using an Ed25519 key pair with password login and root password login disabled.   (Tools & protocols: OpenSSH, Ed25519, sshd, public-key authentication.)

Step-by-step

Generate an Ed25519 key pair

ssh-keygen -t ed25519 -N '' -f ~/.ssh/lab_ed25519

Install the public key as authorised

mkdir -p ~/.ssh && chmod 700 ~/.ssh

Test key-based login

ssh -i ~/.ssh/lab_ed25519 -o StrictHostKeyChecking=no root@localhost whoami

Disable password authentication

sed -i 's/^#*PasswordAuthentication.*/PasswordAuthentication no/' /etc/ssh/sshd_config

Verify password login is blocked

ssh -o PreferredAuthentications=password -o PubkeyAuthentication=no root@localhost

Test it

Attempting password login returns Permission denied (publickey), confirming key-only access.

Note: Full commands and expected output are in labs/lab-23-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 24 — Site-to-Site VPN with WireGuard

Exam objective: build a site-to-site vpn tunnel.

Goal: The learner builds a WireGuard tunnel between two Linux network namespaces acting as two VPN endpoints.

What you'll build

A working WireGuard tunnel between site-a and site-b namespaces with peer keys, allowed-ips and connectivity verified by ping.   (Tools & protocols: WireGuard, wireguard-tools, Curve25519, ip netns.)

Step-by-step

Install WireGuard

apt update && apt install -y wireguard wireguard-tools

Create two namespaces for the two sites

ip netns add site-a

Generate key pairs for each peer

wg genkey | tee a.key | wg pubkey > a.pub

Configure the WireGuard interfaces

ip -n site-a link add wg0 type wireguard

ip -n site-b link add wg0 type wireguard

ip -n site-a addr add 10.99.0.1/24 dev wg0

ip -n site-b addr add 10.99.0.2/24 dev wg0

ip netns exec site-a wg set wg0 \

private-key /tmp/a.key listen-port 51820 \

peer "$(cat /tmp/b.pub)" allowed-ips 10.99.0.2/32

ip netns exec site-b wg set wg0 \

private-key /tmp/b.key listen-port 51821 \

peer "$(cat /tmp/a.pub)" allowed-ips 10.99.0.1/32 \

endpoint 127.0.0.1:51820

Bring tunnels up and test connectivity

ip -n site-a link set wg0 up

Inspect the tunnel state

ip netns exec site-a wg show

Clean up namespaces and keys

ip netns del site-a

Test it

ip netns exec site-b ping -c2 10.99.0.1 succeeds, proving the encrypted tunnel is up.

Note: Full commands and expected output are in labs/lab-24-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 25 — Port Scanning and Hardening

Exam objective: scan ports and reduce attack surface.

Goal: The learner uses nmap to enumerate exposed services on the host and disables the unnecessary ones.

What you'll build

An attack-surface audit produced with nmap and ss, followed by disabling an unneeded service and confirming closure.   (Tools & protocols: nmap, ss, systemctl, TCP, SYN scan.)

Step-by-step

Install nmap

apt update && apt install -y nmap

Scan all 65535 TCP ports on yourself

nmap -sS -p- 127.0.0.1

Enumerate service versions

nmap -sV -p 22,80,443 127.0.0.1

OS fingerprint the host

nmap -O 127.0.0.1

Cross-check open ports with ss

ss -tulnp

Disable an unneeded service

systemctl status apache2 2>/dev/null && systemctl disable --now apache2

Re-scan to confirm the port is closed

nmap -sS -p 80 127.0.0.1

Test it

The re-scan with nmap -sS -p 80 shows the disabled service's port is no longer open.

Note: Full commands and expected output are in labs/lab-25-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 26 — Suricata IDS

Exam objective: detect intrusions with an ids.

Goal: The learner installs Suricata, updates its rule set, triggers a test signature, and inspects the resulting alert.

What you'll build

A running Suricata IDS with the Emerging Threats ruleset that logs an alert for benign test traffic.   (Tools & protocols: Suricata, jq, Emerging Threats rules, IDS/IPS.)

Step-by-step

Install Suricata and jq

apt update && apt install -y suricata jq

Update the Emerging Threats ruleset

suricata-update

Start Suricata on the active interface

suricata -i eth0 -D

Trigger a test signature

curl -s http://testmynids.org/uid/index.html >/dev/null

Inspect the alert log

tail /var/log/suricata/fast.log

Stop Suricata

pkill suricata

Test it

fast.log and eve.json show a GPL ATTACK_RESPONSE alert triggered by the test traffic.

Note: Full commands and expected output are in labs/lab-26-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 27 — ACL with iptables

Exam objective: build a network acl.

Goal: The learner builds an iptables ACL that permits MySQL traffic from one subnet and denies it from everywhere else.

What you'll build

An ordered iptables INPUT chain that accepts port 3306 from 10.0.0.0/24, drops it elsewhere, and is persisted across reboots.   (Tools & protocols: iptables, iptables-persistent, netcat, TCP, ACL.)

Step-by-step

Inspect current rules

iptables -L INPUT -n -v --line-numbers

Allow MySQL from 10.0.0.0/24 only

iptables -I INPUT -p tcp --dport 3306 -s 10.0.0.0/24 -j ACCEPT

Drop MySQL from everywhere else

iptables -A INPUT -p tcp --dport 3306 -j DROP

Verify rule ordering

iptables -L INPUT -n -v --line-numbers

Test the ACL

nc -zv 127.0.0.1 3306        # blocked unless your IP is in 10.0.0.0/24

Persist the rules

apt install -y iptables-persistent

Clean up the rules

iptables -D INPUT -p tcp --dport 3306 -s 10.0.0.0/24 -j ACCEPT

Test it

A connection from source 10.0.0.5 behaves differently to other sources, proving the ACL filters by source.

Note: Full commands and expected output are in labs/lab-27-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

# Domain 5.0 — Network Troubleshooting  (24%)

Methodology · Layer 1/2 Diagnostics · Connectivity Tools · DNS/Routing/MTU · Packet Capture

Key concepts

Follow the seven-step CompTIA methodology: identify the problem → theory → test → plan → implement → verify → document.

Layer 1/2 issues (cabling, interference, attenuation, port/interface errors) are diagnosed with cable testers and interface counters.

Connectivity tools — ping, traceroute / mtr, arp, ip / ifconfig, netstat — isolate faults by OSI layer.

DNS, routing and MTU / PMTUD problems are resolved with dig / nslookup, route inspection and tcpdump / tshark packet capture.

## Lab 28 — Layer 1/2 Diagnostics

Exam objective: diagnose layer 1 and 2 faults.

Goal: The learner diagnoses physical and data-link faults using ip, ethtool and kernel interface counters.

What you'll build

A repeatable Layer 1/2 diagnostic workflow reading link state, speed/duplex and error counters   (Tools & protocols: ip, ethtool, iproute2.)

Step-by-step

Install ethtool

apt update && apt install -y ethtool

Check operational state

ip -brief link show

Inspect link parameters

ethtool eth0

Check error counters

ethtool -S eth0 | grep -iE 'err|drop|crc|frame'

Bounce the interface

ip link set eth0 down

Force speed/duplex (example only, do not run)

# ethtool -s eth0 speed 1000 duplex full autoneg off

Test it

Verify UP link state with clean CRC/frame counters confirms a healthy Layer 1/2 path.

Note: Full commands and expected output are in labs/lab-28-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 29 — ping, traceroute, mtr

Exam objective: locate loss and latency along a path.

Goal: The learner measures reachability, hop-by-hop path and continuous loss/latency using ping, traceroute and mtr.

What you'll build

A path-diagnosis toolkit measuring RTT, per-hop routers and where loss begins   (Tools & protocols: ping, traceroute, mtr, ICMP, TCP.)

Step-by-step

Install the tools

apt update && apt install -y traceroute mtr-tiny

Baseline reachability with ping

ping -c5 8.8.8.8

Trace the hop-by-hop path

traceroute 8.8.8.8

Combined live view with mtr

mtr -rwc 20 8.8.8.8

Use TCP traceroute when ICMP is blocked

traceroute -T -p 443 example.com

Test it

Verify the mtr Loss% column pinpoints where persistent loss starts along the path.

Note: Full commands and expected output are in labs/lab-29-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 30 — DNS Troubleshooting

Exam objective: isolate name resolution problems.

Goal: The learner isolates DNS resolution problems using dig, +trace and resolvectl.

What you'll build

A DNS triage workflow comparing resolvers, walking the delegation chain and decoding error statuses   (Tools & protocols: dig, resolvectl, systemd-resolved, dnsutils.)

Step-by-step

Install dnsutils

apt update && apt install -y dnsutils

Confirm which resolver the system uses

cat /etc/resolv.conf

Compare two upstream resolvers

dig @1.1.1.1 example.com +short

Trace the recursive resolution chain

dig +trace example.com

Check for slow upstream

time dig @1.1.1.1 example.com

Flush systemd-resolved cache

resolvectl flush-caches

Decode common DNS error statuses

Test it

Verify a resolver returning an IP versus NXDOMAIN identifies the failing DNS server.

Note: Full commands and expected output are in labs/lab-30-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 31 — ARP and Neighbour Table

Exam objective: troubleshoot address-to-mac resolution.

Goal: The learner inspects and manipulates the ARP and IPv6 neighbour tables to resolve stale or duplicate entries.

What you'll build

Neighbour-table diagnostics probing the gateway, detecting duplicate IPs and pinning static entries   (Tools & protocols: ip neigh, arping, ndisc6, ARP, ICMPv6 NDP.)

Step-by-step

View the current neighbour table

ip neigh

Probe a neighbour with arping

arping -I eth0 -c2 $(ip route | awk '/default/ {print $3}')

Detect duplicate IP

arping -D -I eth0 -c2 $(ip -4 -o addr show eth0 | awk '{print $4}' | cut -d/ -f1)

Add a static ARP entry

ip neigh add 10.0.0.99 lladdr de:ad:be:ef:00:01 dev eth0

Flush the table

ip neigh flush all

Explore IPv6 neighbour discovery

ip -6 neigh

Test it

Verify arping -D reports a duplicate when another host claims the same IP address.

Note: Full commands and expected output are in labs/lab-31-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 32 — Packet Capture (tcpdump/tshark)

Exam objective: capture and analyse packets.

Goal: The learner captures DNS packets to a pcap and analyses them with tshark and the browser-based TIS PCAP Analyzer.

What you'll build

A saved .pcap of DNS traffic analysed on the CLI and graphically in the TIS PCAP Analyzer   (Tools & protocols: tcpdump, tshark, TIS PCAP Analyzer, DNS, BPF.)

Step-by-step

Install both tools

apt update && apt install -y tcpdump tshark

Capture 10 DNS packets from this host

tcpdump -i eth0 -nn -w /tmp/dns.pcap 'udp port 53 and src host 127.0.0.1' -c 10 &

Read the file with tshark

tshark -r /tmp/dns.pcap -Y "dns" -T fields -e frame.time_relative -e dns.qry.name -e dns.qry.type

Filter only response packets

tshark -r /tmp/dns.pcap -Y "dns.flags.response==1"

Generate a friendly summary

tcpdump -nn -r /tmp/dns.pcap | head

Download the .pcap via base64

base64 /tmp/dns.pcap > /tmp/dns.pcap.b64

Analyse the capture in the TIS PCAP Analyzer at https://alfredang.github.io/pcapanalyzer/

Answer the quick analysis questions

Test it

Verify the TIS PCAP Analyzer shows all three queries with matching responses and transaction IDs.

Note: Full commands and expected output are in labs/lab-32-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 33 — MTU and Path-MTU Discovery

Exam objective: diagnose mtu and pmtud failures.

Goal: The learner tests interface MTU and simulates Path MTU Discovery using the Don't Fragment ping flag.

What you'll build

MTU tests proving where oversized frames blackhole and how MSS clamping rescues TCP   (Tools & protocols: ping, ip link, ICMP, PMTUD.)

Step-by-step

See your interface MTU

ip -brief link show eth0

Send a packet at the largest size that fits

ping -M do -s 1472 -c2 8.8.8.8

Force fragmentation needed

ping -M do -s 1500 -c2 8.8.8.8

Lower the interface MTU and retest

ip link set eth0 mtu 1400

Restore the MTU

ip link set eth0 mtu 1500

Understand MTU-mismatch blackholes and MSS clamping

Test it

Verify a DF ping larger than the MTU fails while a fitting payload succeeds.

Note: Full commands and expected output are in labs/lab-33-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 34 — Routing Issues

Exam objective: diagnose and fix routing problems.

Goal: The learner inspects the routing table, queries route selection and adds or removes static routes.

What you'll build

Routing diagnostics using ip route get, longest-prefix match and static-route add/remove   (Tools & protocols: ip route, traceroute, mtr, iproute2.)

Step-by-step

Print the full routing table

ip route

Ask the kernel which route a destination uses

ip route get 8.8.8.8

Add a static route

ip route add 10.99.0.0/24 via $(ip route | awk '/default/ {print $3}') metric 100

Show route metrics

ip route show 10.99.0.0/24

Detect asymmetric routing

traceroute 8.8.8.8

Remove the test route

ip route del 10.99.0.0/24

Investigate route precedence

Test it

Verify ip route get returns the interface, source IP and next-hop chosen for a destination.

Note: Full commands and expected output are in labs/lab-34-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 35 — Network Emulation with tc

Exam objective: emulate delay jitter loss and bandwidth.

Goal: The learner injects delay, jitter, loss and bandwidth limits on an interface with tc netem and tbf.

What you'll build

A reproducible WAN emulation adding delay, jitter, loss and a bandwidth cap for app testing   (Tools & protocols: tc, netem, tbf, iperf3, sch_netem.)

Step-by-step

Baseline ping

ping -c5 8.8.8.8

Add 200 ms of delay

tc qdisc add dev eth0 root netem delay 200ms

Add jitter

tc qdisc change dev eth0 root netem delay 200ms 50ms distribution normal

Add packet loss

tc qdisc change dev eth0 root netem delay 200ms loss 5%

Limit bandwidth with a token bucket filter

tc qdisc add dev eth0 root tbf rate 1mbit burst 32kbit latency 400ms

Remove all shaping

tc qdisc del dev eth0 root

Test it

Verify ping RTT rises by ~200 ms and iperf3 throughput caps at 1 Mbit/s under shaping.

Note: Full commands and expected output are in labs/lab-35-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

## Lab 36 — Service Reachability Triage

Exam objective: triage service reachability by layer.

Goal: The learner isolates whether TCP, TLS, HTTP or the app is failing using curl, nc and openssl.

What you'll build

A layered site-down triage testing TCP, TLS, HTTP and per-phase request timing   (Tools & protocols: curl, nc, openssl, netcat-openbsd, TCP, TLS, HTTP.)

Step-by-step

Install the tools

apt update && apt install -y curl netcat-openbsd openssl

Test Layer 4 reachability (TCP handshake)

nc -zv example.com 443

Test the Layer 6 TLS handshake

echo | openssl s_client -connect example.com:443 -servername example.com 2>/dev/null | head -20

Test Layer 7 HTTP

curl -v https://example.com 2>&1 | head -30

Inspect timing breakdown

curl -w "DNS:%{time_namelookup} TCP:%{time_connect} TLS:%{time_appconnect} TTFB:%{time_starttransfer} TOTAL:%{time_total}\n" -o /dev/null -s https://example.com

Apply the layered decision tree

Test it

Verify each layer's tool isolates the failing stage from TCP up to the application.

Note: Full commands and expected output are in labs/lab-36-*.md. Reset the Killercoda playground before the next lab if this one changed firewall, routing or interface state.

# Exam Preparation

First pass: do every lab on Killercoda, reading the explanation under each step.

Second pass: redo the labs from memory until the command verbs are automatic.

Memorise the seven OSI layers, the common ports (20/21/22/23/25/53/67-68/80/443…) and the seven troubleshooting steps.

The N10-009 exam is up to 90 questions in 90 minutes; the passing score is 720 on a 100–900 scale.

Take the Tertiary Infotech Network+ practice exam: https://exams.tertiaryinfotech.com/practice-exams/comptia/comptia-network-plus

Book the exam through Pearson VUE from your CompTIA account.

# Quick Command Reference

ip -brief addr — Show interfaces, state and IPv4/IPv6 addresses.

tcpdump -i eth0 -nn — Capture packets on an interface without name resolution.

dig / nslookup — Query DNS records (A, AAAA, MX, NS, PTR…) for troubleshooting.

ip route / route -n — Show the routing table and the selected next hop.

ping / traceroute / mtr — Test reachability, path and per-hop latency.

arp -n / ip neigh — Show the IP-to-MAC neighbour (ARP) table.

ss -tulpn / netstat -tulpn — List listening TCP/UDP services and their ports.

nmap -sT <host> — Discover open TCP ports on a target.

ufw / iptables — Configure the host firewall and packet-filter ACLs.

systemctl status <svc> — Check whether a network service is running.

# Glossary

OSI model — A seven-layer reference model (Physical → Application) describing network communication.

Encapsulation — Adding a layer's header (and trailer) to data as it moves down the stack.

Subnet mask / CIDR — Bits that split an IP address into network and host portions (e.g. /24).

IPv6 SLAAC — Stateless Address Autoconfiguration — a host derives its own IPv6 address.

Port — A 16-bit number identifying a service (e.g. 80 = HTTP, 443 = HTTPS).

DNS — Domain Name System — resolves names to IP addresses via a hierarchy of zones.

DHCP / DORA — Dynamic Host Configuration Protocol; leases addresses via Discover-Offer-Request-Ack.

VLAN / 802.1Q — A logically separate LAN carried over a shared switch using a VLAN tag.

Trunk — A switch link that carries traffic for multiple tagged VLANs.

Link aggregation / LACP — Bonding several links into one for bandwidth and redundancy.

NAT / PAT — Network/Port Address Translation — maps private addresses to public ones.

Administrative distance — A rank that a router uses to choose between routing sources.

SNMP — Simple Network Management Protocol — polls and traps device health via MIBs.

NetFlow — A protocol that exports summarised traffic-flow records for analysis.

Syslog / SIEM — Centralised logging; a SIEM correlates events for security monitoring.

NTP — Network Time Protocol — synchronises device clocks.

CIA triad — Confidentiality, Integrity, Availability — the goals of security.

Firewall / ACL — Rules that permit or deny traffic by address, port or protocol.

IDS / IPS — Intrusion Detection/Prevention System — inspects traffic for threats.

TLS — Transport Layer Security — encrypts data in transit (HTTPS).

VPN — Virtual Private Network — an encrypted tunnel (site-to-site or client-to-site).

MTU / PMTUD — Maximum Transmission Unit and Path-MTU Discovery for packet sizing.

# Support

Training provider: Tertiary Infotech Academy Pte Ltd (UEN 201200696W).

Contact: enquiry@tertiaryinfotech.com · +65 6100 0613 · www.tertiarycourses.com.sg

Course materials and the assessment are on the LMS: https://lms-tms.tertiaryinfotech.com/

Assessment flow

On assessment day, follow this order:

TRAQOM — scan the TRAQOM QR code on the LMS and complete the survey.

Take the Assessment digital attendance.

Sit the Written Assessment (WA), then the Practical Performance (PP) — both open book.

Submit your completed answers on the LMS.

Sign the Assessment Summary Record.